ITP 348: Robotics Chassis Setup

Items You will Need:

- Screwdriver

- 2 DC Gear Motors

- 1 Ball Caster and 1 Hex Jam Nut

- 4 Standoffs and 8 Small Screws

- Bottom and Top Plates with DC Motor Brackets

- 2 Wheels and 2 Screws

- 4 Long Screws and 4 Hex Jam Nuts

2.

2.

1.

1.

3.

3.

5.

5.

4.

4.

6.

6.

7.

7.

STEP 1:

Screw the Ball Caster to the Bottom Plate

STEP 2:

Screw on 3 of the Standoffs to the Bottom Plate using 6 of the screws.

Top View

Bottom View

STEP 3:

Screw the DC Motor Brackets to the DC Motors using the Long Screws and Hex Nuts

STEP 4:

Align the Top Plate with DC Motor Brackets and Standoffs attached to the Bottom Plate

STEP 5:

Screw the Top Plate to the standoffs.

Note: Make sure DC Motor wires are on the inside of the chassis pointing inwards not outward

STEP 6:

Screw on Wheels to DC Motors and add the wheel rubber tires.